Construction-associated solids loads with a temporary sediment control BMP.

Theodore G. Cleveland,1 and Adebola Fashokun2

# Abstract

A highway construction site was monitored to determine the effectiveness of a temporary sediment control (rock filter dam) that was part of the pollution prevention plan that protected storm water leaving the site.  Selected water quality parameters were monitored along with solids specific parameters.  The results were compared in a before-during-after approach and an upstream-downstream approach using a two-sample t-test for differences in the mean values during the different construction phases or locations.

Construction activity caused a six fold increase in total solids leaving the construction site during construction, as compared to pre-construction values.  Construction activity had an effect on the distribution of particles in suspension leaving the construction site.  The solids control device, a rock-filter dam, had an effect on the particle size distribution of suspended particles, but in upstream-downstream analysis did not produce a significant difference in solids leaving the construction site.

1Associate Professor, Department of Civil and Environmental Engineering University of Houston, Houston, Texas 77204-4791

2 Assistant Engineer, CivilTech Engineering Inc., 11821 Telge Road, Cypress, Texas 77429

# KEY TERMS

Storm water; pollution prevention; sediment control; water quality

# INtroductioN

The Clean Water Act (CWA) prohibits storm water discharge from highway construction sites larger than a prescribed area, unless authorized by a National Pollutant Discharge Elimination System (NPDES) permit.  Permittees provide a site description, identify sources of storm water contaminants, identify appropriate pollution control measures, and implement these measures.  Collectively the site description and measures are known as the facility’s Storm Water Pollution Prevention Plan (SW3P).

The permit does not require performance measures, but states that total suspended solids (TSS) can be used as an indicator parameter to characterize the control of other pollutants commonly found in storm water discharges. Therefore, solids control has been the focus of many SW3Ps.

Structural performance monitoring was conducted by the City of Austin (1990).  They concluded that sand filtration and wet pond detention were effective controls for their conditions.  These results are consistent with findings of other researchers (Urbonas and Stahre, 1993; Shaver, 1993).  However these BMPs are for permanent facilities (such as an operating highway) and not for the short-term disturbances caused by construction.  Structural controls were evaluated during construction by Barrett et al. (1995) but focused on sediment control fences.

Although the permit requires SW3Ps to be in-place, it does not require any monitoring.  Should monitoring become required, where, when, and what to measure will become important components of these SW3P technologies.

STUDY DESCRIPTION

This research monitored a drainage ditch adjacent to a 2.3 mile construction site along NASA Rd. 1 in Harris County, Texas, to determine what effect highway construction activities have on solids transport to receiving waters and to evaluate the performance of a rock-filter dam as a pollution prevention device.   Figure 1 indicates the location of the project in relation to Houston, Texas.

[Figure 1 near here]

The ditch was monitored before, during, and after highway construction.  Two locations in the ditch were monitored: one immediately upstream of a rock-filter dam temporary sediment control (TSC), another about 300 feet downstream of the TSC location.  The construction project was to widen the road. The soil-disturbing activities included preparing the right-of-way, grading, excavation, storm sewer placement, utility adjustments, and topsoil replacement.

Figures 2 and 3 are images of the upstream TSC location prior to construction activity and during construction activity.  In these images the utility pole foundation along the roadway and the electricity riser indicate that both images are of essentially the same location even though the image angle is slightly different. The direct exposure of bare-earth is obvious in these contrasting images.

[Figure 2 near here] [Figure 3 near here] {Side-by-side presentation if possible}

The TSC operates by interception of solids laden storm water from disturbed areas, coincident reduction of flow speed, sedimentation and capture of the solids, and release of the water in a sheet flow (TxDOT, 1993).   Although the TSC is called a rock-filter-dam, classical filtration (size exclusion, interception in the media, and cake formation) is not the intent of these devices.

METHODS

The TSC evaluation was based on results of chemical and physical analyses from 142 sampling visits, from March 25, 1997, through July 9, 1998; about every three days.  Of these visits, 27 during pre-construction, 58 during construction, and 11 post-construction yielded paired storm and non-storm samples.  Some sample visits did not collect storm samples because there were no recent storms before the visit, but all sample visits collected a non-storm sample.  Storm samples were collected using a ball-valve type 3-liter mechanical sampler.  The storm sampler is a first-flush-type sampler and collected samples from runoff producing rainfall during the study period.  Non-storm samples were grab samples adjacent to the mechanical sampler.

Rainfall was measured on-site using a tipping-bucket rain gage with a cumulative depth data logger.   Rainfall from this on-site gage is compared in Figure 4 to a nearby continuous gage about one-mile away operated by the Harris County Office of Emergency Management (HCOEM, 2005).  The cumulative rainfall pattern at each gage is about the same, exact agreement is not expected.  The average rainfall intensities were 0.007 in/hr., 0.004 in/hr., and 0.009 in/hr for the pre-construction, during-construction, and post-construction monitoring phases.  The largest incremental rainfall depths in the three phases were about 3-in., 5-in., and 4-in.  These summary statistics indicate that the rainfall behavior is about the same for the three construction phases.  The authors estimate that a 0.4-inch incremental rainfall depth produced enough runoff to fill the storm sampler.

[Figure 4 near here]

Table 1 is a list of the solids and water quality measures collected during this research.  These data were categorized as solids measures (turbidity, TSS, TS), nutrient measures (NO3, NO2, NH4, PO4), and solids size characteristics (D10, D50, D90, and the fraction smaller than 75 m).  The particle-size data were measured by a laser-diffraction instrument that assumed spherical particles.

[Table 1 near here]

The non-storm samples were compared to storm samples during the three activity phases.   Upstream and downstream comparisons were made during the construction phase.  These comparisons were made using the mean value for a particular parameter from all data collected during that activity phase.  The differences in these mean values were considered statistically significant if the p-value for a two-sample t-test statistic associated with the particular pair of means was smaller than 0.05.

RESULTS

The results from these analyses are summarized in Table 2.  The results are stratified by activity phase, with the during-construction phase sub-divided into upstream and downstream to reflect the presence of the TSC during construction.  Of the parameters analyzed only TSS (and to a lesser extent TS), PO4 and the size characteristics had statistically meaningful differences.

[Table 2 near here]

TSS from first-flush samples is more than three times larger than in non-storm samples in the pre-construction phase.  PO4 is about two times larger in the first-flush samples in this phase.  The other parameters did not exhibit significant differences.

TSS from first-flush samples is about ten times larger than in non-storm samples in the during- construction phase.  The size characteristics in first-flush samples differ in upstream-downstream analysis, as does PO4.  The other parameters did not exhibit significant differences.  The TSS in first-flush samples in this phase of construction is about six-times larger than in the pre-construction phase.

TSS from first-flush samples is ten times larger than in non-storm samples for the post-construction phase.  The other parameters did not exhibit significant differences in this phase.  The TSS in first-flush samples in this phase of construction is not significantly different than in the pre-construction phase.

TSS from first-flush samples in the during-construction phase is the same upstream and downstream of the TSC.  Only PO4 and size characteristic differences are significant in upstream-downstream analysis.   The non-storm size characteristics are coarser as compared to the pre-construction non-storm samples, but finer for the storm samples upstream of the TSC.

Figure 5 is a plot of the size characteristics represented as a distribution.  The active-construction upstream storm data having finer sizes as compared to the non-storm samples is different than either the pre-construction or post-construction behavior.  The cause for this difference is unknown, possibly some change in source distribution, or re-suspension of previously deposited smaller particles.  The upstream TSS during storm conditions is less than downstream, which might support the re-suspension hypothesis, except these differences are insignificant.  The non-storm samples demonstrate that the source distribution during construction has changed, but to coarser sizes.  The particle size distribution measures increasing downstream of the TSC suggests that the TSC is having an effect on the solids. The smaller sizes remaining in suspension upstream are expected as the flow velocities are supposed to be reduced by the TSC (and coarser sized particles should be removed).  This distribution shift is similar to those that have been observed in engineered storm water treatment systems (Pitt, et al. 1999).

CONCLUSIONS

Construction activity increases the TSS concentrations, about double for non-storm samples and about six times larger for storm samples.  These increases are consistent with findings of other studies, but of lower magnitude (eg. Wolman and Schick, 1967; Vice et al., 1969).   The post-construction samples indicated that the storm water quality is returning to pre-construction behavior as expected.

TSS exhibited no statistically significant difference in upstream-downstream analysis.  The particle size distribution increased downstream of the TSC.  This distribution shift, combined with the PO4 reduction moving downstream, and combined with field-notes of solids residuals trapped upstream of the TSC suggest that some benefit was conferred by the TSC, but this benefit was not quantified.

The TSC studied in this research failed as a pollution control device for controlling TSS.   The authors note that during post-storm visits of the TSC large volumes of solids were deposited upstream of the TSC that were subsequently removed by the construction company as per the SW3P.  These solids likely would have discharged to the ditch without the TSC in place.   For rock-filter dams and similar controls, future research should include measurements of the volume or mass of solids captured upstream of the TSC after a storm event, and report this volume or mass as an estimate of the solids removed by the TSC.  These volumes or masses would demonstrate some benefit of the TSC as a pollution control device and justify their continued use.  Water quality sampling alone may not.  Characterization of these residual solids should provide improved insight into the ability of such TSCs to provide pollution control.

# ACKNOWLEDGEMENTS

The research described in this paper was supported by the Texas Department of Transportation under project number 0-1425.  The discussions and guidance of the project director James Darden, and the Area Engineer, Maureen Wakefield is gratefully acknowledged.  The contents of this paper reflect the views of the authors, who are responsible for the facts and accuracy of the data presented herein.  The contents do not necessarily reflect official views or policies of the U.S. Department of Transportation, Federal Highway Administration or the Texas Department of Transportation.  This paper does not constitute a standard, specification, or regulation.

# REFERENCES

Austin, City of. 1990. “Stormwater Pollutant Loading Characteristics for Various Land Uses in the Austin Area.” Final Report, Environmental Resource Management Division, Environmental and Conservation Services Department, Austin, Texas.

Barret, M.E., J.F. Malina Jr., R.J Charbeneau, G.H. Ward. 1995.  Effects of Highway Construction and Operation on Water Quality and Quantity in an Ephemeral Stream in the Austin, Texas Area, Center for Research in Water Resources, Technical Report 262.  The University of Texas, Austin, TX.

Barrow, N. J. 1983. A mechanistic model for describing the sorption and desorption of phosphate by soil.  J. of Soil Science, 34. pp 733-750.

Fetter, C.W. 1999. “Contaminant Hydrogeology – 2nd Edition” Prentice Hall, NJ. 500p

Hach Company. 1992. “Water Analysis Handbook.” Hach Company, Loveland, CO.

Harris County Office of Emergency Management. 2005. <http://www.hcoem.org/archives.htm>

Pitt, R., B. Robertson, P. Barron, A. Ayyoubi, and S. Clark. 1999. Stormwater Treatment at Critical Areas: The Multi-Chambered Treatment Train (MCTT). U.S. Environmental Protection Agency, Wet Weather Flow Management Program, National Risk Management Research Laboratory. EPA/600/R-99/017. Cincinnati, Ohio. 505 pgs.

Shaver, E. 1993. “Beach Community Adds Sand Filters to Storm Drains,” Water Environment and Technology, 5(5), 18.

Texas Department of Transportation (TxDOT). 1993. Storm Water Management Guidelines for Construction Activities. Texas Department of Transportation, Austin, TX.

Texas Natural Resources Information System (TNRIS). 2005. < http://www.tnris.org/>

Urbonas B.R., and P. Stahre. 1993. STORMWATER Best Management Practices and Detention for Water Quality, Drainage, and CSO Management.  Prentice Hall, Englewood Cliffs, N.J.

Vice, R.B., H.P. Guy, and G. E. Ferguson. 1969. Sediment Movement in an Area of Suburban Highway Construction, Scott Run Basin, Fairfax County, Virginia, 1961–1964, Geological Survey Water Supply Paper 1591-E, USGS, Reston, VA.

Wolman, M.G. and A. P. Shick. 1967. Effects of Construction on Fluvial Sediment; Urban and Suburban Areas of Maryland. Water Resources Research 3: 451-464.