CHAPTER 5   SYSTEM DEVELOPMENT

# 5.1	Overview

The development of the Internet-based system is described in this chapter. The basic design requirements are described in the second section of this chapter. The hardware, and software selections of the system are defined in the third section of this chapter. Components of the system are described in the fourth section. The guidance modules are presented in the fifth section along with some screenshots. The concept of model visualization is presented in the sixth section. A demonstration session is shown in the last section to illustrate how one works with the system from a user’s perspective.

# 5.2	Basic Design Requirements

The ANTS system must contain the following features in order to be fully functional:

- guidance modules to help a user in selecting a most appropriate analytical contaminant transport model from a model library to predict the outcome of a contaminant transport problem,

- a model library that contains a large collection of analytical solutions of contaminant transport models, and

- ability to be used with or without much domain and computer knowledge.

# 5.3	Hardware and Software Selections

The Internet-based system has two parties involved: a web server and a web client. Figure 5-1 shows the architecture of the ANTS system. The web server has been provided in HTML, CGI, and Java. The web client can be any web browser. The model database was created by using Microsoft Access.

Figure 5-1  The ANTS architecture

## 5.3.1	Web Server

A web server is a program that monitors for connections from a web browser, gets the information about the calling machine, searches the server configuration files to see if it has any instructions about accepting or denying requests from the machine that is calling, and acts accordingly. The hardware and software selections for a web server machine are the following:

- Pentium class CPU,

- 32 MB RAM (minimum),

- 100 MB hard disk storage space,

- fast Internet connection (such as LAN),

- Microsoft Windows NT 4.0 (or a higher version),

- Microsoft Internet Information Server (or Netscape FastTrack Server),

- CGI component (such as Perl or Microsoft FrontPage server extensions), and

- Open Database Connectivity (ODBC).

## 5.3.2	Client

Any online computer equips with a Java-enabled web browser should be able to use the Internet-based modeling guidance system. The two preferred web browsers for which the system is tested are Netscape Navigator for Windows Version 3.0 (or higher) and Microsoft Internet Explorer for Windows Version 3.0 (or higher).

# 5.4	Components of the System

The Internet-based system has four major components: a web-based graphical user interface, databases, a model library, guidance modules, and visualization modules.

## 5.4.1	Graphical User Interface Component

In order to effectively use the Internet-based system to achieve a goal, a user must interact with the system via the graphical user interface component. Designing a user-friendly graphical user interface is an important task in building the system. The graphical user interface in the system is a combination of many HTML web pages. This design allows the Internet-based system to have a universal appearance under different platforms. The web pages are classified into eight groups. The organization of the web pages is shown in Figure 5-2.

Figure 5-2  The structure of the web pages in the system

- The index web pages include the home page, introduction pages, content pages, and list pages.

- The documentation pages include model documents, model references, and tutorial pages. The tutorial pages contain information regarding basic concepts of contaminant transport modeling.

- The Java model pages allow users to use online Java analytical solution models for solving contaminant transport problems.

- The non-Java model pages allow users to download non-Java analytical models (such as the Microsoft Excel worksheets or the FORTRAN programs).

- The modeling guidance pages are interactive guidance pages to help the user choose a model.

- The model visualization pages display visual scenes of model characteristics.

- The software archive pages allow users to download software available in the ANTS web site.

- The navigation bar allows users to link to any web page in the system.

## 5.4.2	Database Component

The model database stores information regarding the properties of all models. The database was created by using Microsoft Access.

## 5.4.3	Model Library Component

The model library has a large collection of analytical solution models in three different formats:

- The spreadsheet models: Twenty-two models are available as Microsoft Excel worksheets. These model worksheets can be downloaded from the ANTS web site.

- The FORTRAN models: More than 60 models are available either as standalone Microsoft Windows applications or FORTRAN source codes. These models can be downloaded from the ANTS web site.

- The Java applet models: More than 100 models are compiled as Java applets to be used via the Internet.

Most spreadsheet models are available as the FORTRAN models as well as the Java applet models. Most FORTRAN models are available as the Java applet models.

The model library not only consists of one-dimensional single chemical constituent models that are most commonly used in classrooms, laboratory, or fields, it also includes some multidimensional, multisite, multiregion, or heterogeneous media models that have these distinguishing characteristics and can be applied on specific problems. The current list of all models is attached in Appendix C.

## 5.4.4	Guidance Module Component

The guidance module component functions as a guide that can point users to an appropriate model. There are several guidance modules available in the system. As mentioned in Chapter 3, each guidance module has its own match strategy for finding an appropriate model. The guidance modules were created by HTML, CGI, and Java. The details of the guidance modules are described in the next section.

## 5.4.5	Visualization Module Component

The model visualization component was designed to help users understand the basic concepts of a particular model or a particular condition. The details of the model visualization component are described in the latter section.

# 5.5	Designs and Structures of Guidance Modules

## 5.5.1	Hierarchy Guidance Module

The hierarchy guidance module that is based on the hierarchy scheme described in Chapter 3 has a ten-layer model hierarchical tree. Ten properties were used to classify analytical solution models. The properties are as follows: Models, Zone, Flow, Dispersion, Media, Component, Initial Conditions, Inflow Boundary Conditions, Outflow Boundary Conditions, and Source Type.

Figure 5-3 shows three screenshots of the hierarchy guidance module. More than 80 analytical solutions of contaminant transport models were classified into the hierarchical model tree. Because the whole model tree is too large to fit in a single page (or a computer screen), the tree was divided into 16 subtrees. A synchronized hierarchical tree that contains 16 subtrees is shown in Figure 5-3(a). A screenshot of a subtree (Subtree 1-1) is shown in Figure 5-3(b).

To use the module, a user just follows the hierarchical tree with particular model properties in his (her) mind to advance to the next layer. The user should find a model in the end point of the tree. There is a help menu bar in every subtree as shown in Figure 5-3(c). The user can click a menu bar item to receive help information.

| (a) | (b) |
| --- | --- |
| (c) |  |

Figure 5-3  Structure of a hierarchical tree and a subtree (subtree 1-1)

## 5.5.2	Database Schemes

### Ranking Module

A screenshot of the ranking module is shown in Figure 5-4. The ranking module has a five-keyword search engine that has a built-in SQL program overlaid by the ranking module web page. To use the module, a user adds keywords into five text boxes. The user then submits the requests by clicking the Submit button. The module will rank all models based on the user’s requests. A ranked list of all models will show on a table in a new web page.

Figure 5-4  A screenshot of the ranking scheme module

### Querying Module

In the querying module, a user writes an SQL program to retrieve information from the model database. An SQL program that produces the same results as the ranking module in Figure 5-4 is shown in Figure 5-5.

UPDATE ModelRanking SET ModelRanking.Score1 = 50 WHERE ModelRanking.Abstract LIKE '%%%textbox1%%%'; UPDATE ModelRanking SET ModelRanking.Score1 = 40 WHERE ModelRanking.Abstract LIKE '%%%textbox2%%%'; UPDATE ModelRanking SET ModelRanking.Score1 = 30 WHERE ModelRanking.Abstract LIKE '%%%textbox3%%%'; UPDATE ModelRanking SET ModelRanking.Score1 = 20 WHERE ModelRanking.Abstract LIKE '%%%textbox4%%%'; UPDATE ModelRanking SET ModelRanking.Score5 = 10 WHERE ModelRanking.Abstract LIKE '%%%textbox5%%%'; UPDATE ModelRanking SET ModelRanking.FinalScore = ModelRanking.Score1 + ModelRanking.Score2 + ModelRanking.Score3 + ModelRanking.Score4 + ModelRanking.Score5;

Figure 5-5  An SQL program

The SQL program is not as complicated as it seems. The user UPDATEs six fields WHERE certain conditions must be met. The ranking module is more user-friendly than the querying module.

## 5.5.3	Scenario Module

The scenario module is a form that is a collection of form fields on a page along with a form handler, a method of collecting information from the form. The form was created by using HTML and CGI. As shown in Figure 5-6, the module contains a series of form fields. Each form field has two parts: a label that contains a scenario question and a drop down menu that stores possible choices for the question. A user answers the questions by selecting options from the drop down menus to match a groundwater contaminant transport problem. The user then submits the form by clicking the “Submit” button. The system will return a list of appropriate models that can be used to simulate the user’s problem.

Figure 5-6  A screenshot of the scenario scheme

One problem in the scenario module is that not all scenario combinations match a model. There are ten scenario questions in the module as shown in Table 5-1. The number of the possible scenario combinations in this module is 23,040. There are only 120 models available in the model library. It is very likely that a user can not find a match model by choosing a particular scenario combination. To minimize this problem, the author suggests the following hints:

- If a user is not sure about a particular question, choose the unknown option to bypass the question. A list of all available models in the database can be found by checking the unknown options to all questions.

- The user should apply only one condition at the beginning of the search. Submit the request to see if any model will meet the scenario. Then add one condition at a time to narrow down the target.

Table 5-1  A list of questions and choices in the scenario module

| Scenario Questions | Choices |
| --- | --- |
| What is the problem type? | - Groundwater contaminant transport |
| - What kind of zone is the aquifer? | - Saturated - Unsaturated |
| - What is the flow field type? | - One-dimensional principal directional flow - One-dimensional arbitrary directional flow - Two-dimensional flow - Radial flow |
| - What is the dispersion type? | - One-dimensional dispersion - Two-dimensional dispersion - Three-dimensional dispersion |
| - What kind of media does the aquifer have? | - Homogeneous medium - Heterogeneous medium - Two-region medium - Layered soil medium |
| - How many chemicals are in the aquifer? | - Single component - Multicomponent |
| - What are initial conditions? | - Constant initial distribution - Decaying initial distribution |
| - What are inflow boundary conditions? | - The first type - The third type |
| - What are outflow boundary conditions? | - Semi-infinite - Finite domain |
| - What is the source type? | - 15 different source types |

# 5.6	Model Visualization

The purpose of the model visualization is to help users understand the basic concepts of a particular model or a particular condition. The first part of the section describes the concepts of the model visualization. Two visualization modules, visual characteristics and visual model mosaic, are also described in this section.

## 5.6.1	Introduction

It is difficult to understand the meaning of a technical term or an equation without illustrations. Just as an example, what is the difference among the first type, the second type and the third type inflow boundary conditions? As the results from the user survey show in Chapter 3, very few respondents can answer this question. This section describes some model characteristics that may be difficult to understand without a graphical representation.

### Inflow Boundary Conditions

The first type, the second type, and the third type inflow boundary conditions are described in this section. The first type inflow boundary condition, or the Dirichlet inflow boundary condition, prescribes the concentration along a portion of the boundary as

C=C0x,y,z,								(5-1)

where Co is a given function of time and space for that particular portion of boundary.

The second type inflow boundary condition, or the Neumann inflow boundary conditions, prescribes the normal gradient of the concentration over a certain portion of the boundary. This boundary conditions can be given as

−Dx¶C¶x=VC0,								(5-2)

where C0 and Dx are the known measured concentration in the influent water and the dispersivity in the flow direction, respectively.

The third type inflow boundary condition, or the Cauchy inflow boundary conditions, prescribes the normal gradient of the concentration over a certain portion of the boundary. The flux of solute across the boundary is dependent upon the difference between a specified concentration value on one side of the boundary and the solute concentration on the opposite side of the boundary. This boundary conditions can be given as

VC−Dx¶C¶x=VC0,								(5-3)

where C0 and Dx are the known measured concentration in the influent water and dispersivity in the flow direction, respectively.

A domain novice is likely to have difficulty in understanding the differences among equations (5-1), (5-2), and (5-3). A conventional approach is to draw a graph to represent each condition. However, it is not easy to illustrate these three conditions in three graphs. A different approach was used in this research by creating a series of graphs to illustrate each condition. Figures 5-7, 5-8, and 5-9 show concentration advances of the first type, the second type, and the third type inflow boundary conditions, respectively. Figure 5-7 displays advection as the only mechanism in the concentration advance of the first type inflow boundary condition. Figure 5-8 displays diffusion as the only mechanism in the concentration advance of the second type inflow boundary condition. Figure 5-9 displays advection and diffusion as two mechanisms in the concentration advance of the third type inflow boundary condition.

Figure 5-7  Concentration advance of the first type inflow boundary

Figure 5-8  Concentration advance of the second type inflow boundary

Figure 5-9  Concentration advance of the third type inflow boundary

### Contamination Source Types

There are 15 contamination source types in the system. It may be difficult to understand a source type without a graphical representation. As an example, a user should be able to distinguish the differences among a one-dimensional instantaneous pulse injection, a one-dimensional continuous constant concentration injection, and a two-dimensional continuous point injection from Figures 5-10, 5-11, and 5-12. Figure 5-10 shows the advance of a one-dimensional instantaneous pulse injection source in a series of graphs. Each graph represents the advance of a pulse injection source in a particular time. Figure 5-11 shows the advance of a one-dimensional continuous constant concentration injection source in a series of graphs. Figure 5-12 shows the advance of a two-dimensional continuous point injection in a horizontal plane.

Figure 5-10  Advance of a one-dimensional instantaneous pulse injection

Figure 5-11  Advance of a one-dimensional continuous constant injection

Figure 5-12  Advance of a two-dimensional continuous point injection

### Contamination Dispersion Type

There are three contamination dispersion types in the system. A user should be able to distinguish the differences among a one-dimensional dispersion, a two-dimensional dispersion, and a three-dimensional dispersion from Figures 5-13, 5-14, and 5-15. Figure 5-13 shows the advance of a one-dimensional dispersion in a series of graphs. Figure 5-14 shows a two-dimensional dispersion in an aquifer. Figure 5-15 shows a three-dimensional dispersion in an aquifer.

Figure 5-13  One-dimensional dispersion

Figure 5-14  Two-dimensional dispersion

Figure 5-15  Three-dimensional dispersion

## 5.6.2	Visual Characteristics

Most web browsers can display animated graphics interchange format (GIF) images. The GIF is a graphical file format commonly used to display indexed color images on the World Wide Web. The animated GIF is a file containing a series of GIF images that are displayed in rapid sequence by most web browsers, giving an animated effect. The visual characteristics take the advantage of the animated GIF image. In this approach, an animated GIF image was used to show a particular scenario such as the advance of a plume in a porous medium. Many animated GIF images were added into the system to help users understand the concept of groundwater contaminant transport. The author is unable to show animated effects of the visual model characteristics in the dissertation. The animated GIF images of the inflow boundary conditions can be found on the following web link:

http://clevleand1.cive.uh.edu/ANTS/Guidances/mosaic/

visual-boundarycondition.html .

The animated GIF images of the contamination source types can be found on the following web link:

http://clevleand1.cive.uh.edu/ANTS/Guidances/mosaic/

visual-sourceterm.html .

The animated GIF images of the contamination dispersion types can be found on the following web link:

http://clevleand1.cive.uh.edu/ANTS/Guidances/mosaic/

visual-dispersion.html .

## 5.6.3	Visual Model Mosaic

The visual model mosaic is used to help users understand the basic concepts of a model. A conventional approach is to draw a graph to represent a model. A different approach developed in this research is to use a visual model mosaic to represent a model. It is possible to represent a model by using the model mosaic that is a combination of visualized characteristics graphs. The major advantages of the mosaic approach are its reusable tiles and its dynamic results. A tile can be used in any model as long as the model has the characteristic. By using HTML and CGI, a model mosaic can be generated dynamically. In other words, a user can create a visual scene by picking parts.

A visual model mosaic is a conceptual graphical scene of a model. A visual model mosaic is a combination of 11 components that are GIFs. Each component represents a particular property of a model. A mosaic has three rows. The first row has three components: source term, initial condition, and model type. The second row has four components: inflow boundary condition, medium type, zone type, and outflow boundary condition. The last row also has four components: component type, dispersion type, flow type, and the legend of the mosaic. The geometric layout of a mosaic is shown in Figure 5-16.

Figure 5-16  The layout of a visual model mosaic

The combination of HTML and visual model mosaics provides a fast and easy way to generate a conceptual scene of a model. An example model mosaic of the J1DVG1014 model (van Genuchten and Alves, 1982) is shown in Figure 5-17.

Figure 5-17  A screenshot of a visual model mosaic

# 5.7	Demonstration Session

A demonstration session is presented to illustrate the general features of the system. The hypothetical problem is a leak of gasoline from a storage tank for a period of 10 days, after which time it was halted and reported. A sand aquifer is assumed to underlie the site. The problem is to estimate the areal extent of dissolved benzene in the ground water at the time the leak was discovered. Figure 5-18 shows a leaking underground storage tank. The plume is moving toward a drinking water supply that is located in 1000 feet east of the underground tank. Local environmental agency needs to find out how long will this plume reach the water supply. The maximum allowable dose is 10 mg/m3.

Figure 5-18  A two-dimensional plane view of the problem

To use the ANTS system, a user first connects the computer to the ANTS web site at

http://cleveland1.cive.uh.edu/ANTS/ants-welcome.html.

Figure 5-19 shows the ANTS welcome web page that should appear on the screen.

Figure 5-19  A screenshot of the ANTS welcome web page

The user can choose to use the Frame version or the No Frame version of the ANTS system by clicking one of the buttons. The difference between the Frame version and No Frame version of the ANTS system is that the Frame version has a navigation bar to help a user browse the web site. If the user chooses the Frame version, the ANTS content window should appear as shown in Figure 5-20. The navigation bar is shown in the left side of the window. The user can use this navigation bar to browse the ANTS web site.

Figure 5-20  A screenshot of the ANTS content web page

Next, The user clicks the ANTS Modeling Guidances item to choose a modeling guidance. The ANTS Model Guidances web page should appear as shown in Figure 5-21.

Figure 5-21  A screenshot of the ANTS guidance web page

Next, the user chooses the Hierarchy Guidance Module Link. The Hierarchy Guidance Modules web page should appear as shown in Figure 5-22. First time users should read the page to understand the concept of the guidance modules.

Figure 5-22  A screenshot of the hierarchy modules web page

Next, the user chooses the Giant Tree module to find an appropriate model. The following web page should appear. Read the page and click the Click here to begin the search!  link to start a search.

Figure 5-23  A screenshot of the synchronized hierarchical tree

To use the guidance, click the 1-1 button to advance to the next layer, as shown in Figure 5-24(a). By using the hierarchy modeling guidance, a possible model search path is shown in Table 5-2. The Java J2DDR01P model is chosen to solve the case. The subtrees involve in the search path is shown in Figure 5-24 series. A screen capture of the Java J2DDR01P model is shown in Figures 5-25 and 5-26.

Table 5-2  A possible search path of the hierarchy modeling guidance

| Steps | Actions | Reasons |
| --- | --- | --- |
| Go to Subtree (1-1) | Choose Subtree (1-1) | The starting point |
| Choose Subtree (2-2) | Click Link to 2-2 button to jump to Subtree (2-2) | A multidimensional dispersion model is needed. |
| Choose Subtree (3-21) | Click Link to 3-21 button to jump to Subtree (3-21) | The aquifer is a homogeneous media. |
| Choose the Java J2DDR01P model | Click J2DDR01P button to activate the model applet | Contaminant source is a continuous source type. |

| (a) (b) | (c) |
| --- | --- |

Figure 5-24  An example search path

Figure 5-25  A snapshot of the Java J2DDR01P model

Figure 5-26  A snapshot of the Java J2DDR01P model

Table 5-3 contains the parameter values for two observation points. By using the J2DDR01P model, the times to reach 100 mg/m3 in the observation points were computed. The results are 285 days and 290 days in the points A and B, respectively. The result data shown in the Figure 5-25 can be extracted to other programs (such as Microsoft Excel) for plotting.

Table 5-3  Parameters and results of the example case

| Parameters | Point A | Point B |
| --- | --- | --- |
| X Location Y Location Aquifer Width C0 Velocity Dx Dy | 1000 ft 0 ft 800 ft 1000 mg/m3 2 ft/day 60 ft2/day 12 ft2/day | 700 ft 300 ft 800 ft 1000 mg/m3 2 ft/day 60 ft2/day 12 ft2/day |
| Time to reach 10mg/m3 | 285 days | 290 days |

There is a help menu bar in every web page of the hierarchical tree, as shown in Figure 5-27.

Figure 5-27  A screenshot of the help menu bar

The help web pages can be accessed by clicking items on the help menu bar. As an example, if the user clicks the flow button on the menu bar, the flow help web page should appear as shown in Figure 5-28.

Figure 5-28  A snapshot of the flow help web page