Evaluation of Impact from Storm Water Discharges Associated with

Road Construction Activities on Receiving Streams

A Thesis

Presented to

the Faculty of the

Department of Civil and Environmental Engineering

University of Houston

In Partial Fulfillment

of the Requirements for the Degree

Master of Science

in Civil Engineering

by

Kriton Theodoridis

August 1998

Evaluation of Impact from Storm Water Discharges Associated with

Road Construction Activities on Receiving Streams

____________________________________

Kriton Theodoridis

Approved:          		  	                   _______________________________

Chairman of the Committee

Theodore G. Cleveland, Associate Professor

Civil and Environmental Engineering

Committee Members:     		       	       _______________________________

Deborah J. Roberts, Associate Professor

Civil and Environmental Engineering

_______________________________

Jerry R. Rogers, Associate Professor

Civil and Environmental Engineering

_________________________		       _______________________________

Charles Dalton, Associate Dean		       Dennis Clifford, Professor and Chairman

Cullen College of Engineering		        Civil and Environmental Engineering

Acknowledgments

I would like to express my gratitude to the Texas Department of Transportation for making this research possible by providing the necessary funds.

I would also like to thank Dr. Ted Cleveland for his patience, support and encouragement since the beginning of this research project.  He has been a good advisor as well as a good friend to me during two quite wearisome years of my life.  Special thanks go to Dr. Debbie Roberts for giving me that first glimpse of research work in my undergraduate years, for always having her door open to me and for always greeting me with a smile.  I would also like to thank Dr. Jerry Rogers for taking the time to serve on my committee.

I want to recognize my fellow students, Luke, Joe, Sam, Bosse, Bola and Harrison for listening to all my complaints and sharing their stories with me.  Special thanks also go out to Leslie and Linda from TNRCC for making this experience both more educating and more fun.

I am very grateful to my brother , for always setting higher standards in my life, my father  for guiding me to a very satisfying career and my best friend over the last five years, Cynthia, for being there for me.

Finally I would like to say thanks to you mom, for all the sacrifices you made that allow me to be standing where I am today.  This is as much of your accomplishment in life and death as it is mine.

Evaluation of Impact from Storm Water Discharges Associated with

Road Construction Activities on Receiving Streams

An Abstract

of a

Thesis

Presented to

the Faculty of the

Department of Civil and Environmental Engineering

University of Houston

In Partial Fulfillment

of the Requirements for the Degree

Master of Science

in Civil Engineering

by

Kriton Theodoridis

August 1998

Abstract

In order to justify the costs involved in installing and maintaining Storm Water Pollution Prevention Plans in road construction activities, a tidal tributary (Cow Bayou), that served as a final receptor of runoff from a road construction, was monitored for a period of 1-1/2 years.  Physical and biological data were collected.  A benthic bioassessment method and a Microtox sediment testing technique were used.  Reduction in total taxa up to 50% in the downstream sampling station at the beginning of the construction suggests that the current measures provide little protection during the early stages of construction. A reduction in total taxa up to 55% in the downstream sampling station during or right after a major rainfall event suggests that these plans provide little relief from the immediate effects of rainstorm.  The Microtox basic solid phase test showed little sensitivity to the type of sediments and pollution levels encountered in this research.

Table of Contents

Acknowledgments								iii

Abstract										v

Table of Contents								vi

List of Figures									viii

List of Tables									x

Chapter 1 Introduction							1

Background									1

Objectives									3

Scope and Limitations								3

Organization									4

Chapter 2 Literature Review						5

Chapter 3 Methodology							14

General Approach								14

Selection of Sampling Location and Development of Sampling Techniques	15

Field Trip Procedures								22

Suspended Solids Analysis							28

Benthic Macroinvertebrate Analysis						29

Microtox Analysis								34

Particle Size and Initial Dilution Experiments					36

Storage Time Experiments							38

Chapter 4 Discussion of Results						40

Water Quality Summary							40

Bioassessment Results								41

Microtox Sediment Test Results						55

Comparison of Benthic Bioassessment and Microtox Sediment

Toxicity Tests								58

Effect of Particle Sizes on the Microtox Basic Solid Phase Test Results		66

Effect of Sample Storage Time on the Microtox Basic Solid Phase

Test Results									73

Chapter 5 Summary and Conclusions					76

Summary									76

Conclusions									76

Recommendations								79

Future Work									81

References									82

Appendix A: Results of Benthic Macroinvertebrate Analysis		86

Appendix B: Results of Microtox Basic Solid Phase Tests		102

Appendix C: Results from Particle Size Analyzer (Mastersizer)	104

Appendix D: Sample Water Quality Data					106

Appendix E: Total Polychaete Taxa and Sheldon’s Evenness Index	107

Appendix F: Rainfall Data							110

Appendix G: Most Commonly Found Benthic Macroinvertebrates

in Cow Bayou							111

List of Figures

Figure 1: USGS map of the construction site and the sampling locations

along Cow Bayou in Clear Lake, Texas					14

Figure 2: Drainage outfalls from the construction site as seen from NASA

Road 1									16

Figure 3: Drainage outfalls from the construction site as seen from Cow

Bayou									16

Figure 4: Column support used as reference for all water measurements

at Station 1									20

Figure 5: Bridge column used as reference for all water measurements

at Station 2									20

Figure 6: Water mark used as reference for all water measurements

at Station 3									21

Figure 7: A graphical presentation of the apparatus used for benthic

macroinvertebrate collection at Cow Bayou				22

Figure 8: Unloading of the canoe at the field site				25

Figure 9: Area where canoe was inserted in Cow Bayou			25

Figure 10: Field data sheet used in all field trips at Cow Bayou			26

Figure 11: Sketches and assigned numbers for the first nine species found

during benthic macroinvertebrate analysis					32

Figure 12: Photographs of representative species taken using the SONY

system during benthic macroinvertebrate analysis				33

Figure 13: Total taxa at every station						47

Figure 14: Diversity (H’) at every station					49

Figure 15: Total monthly precipitation and diversity (H’) for Station 3		54

Figure 16: Microtox EC50 values at every station				56

Figure 17: Total monthly precipitation and Microtox EC50 values for

Station 3									58

Figure 18: Diversity (H’) and Microtox EC50 values for Station 3		60

Figure 19: Total taxa and Microtox EC50 values for Station 3			60

Figure 20: Total taxa versus Microtox EC50 values for all stations		62

Figure 21: Diversity (H’) versus Microtox EC50 values for all stations		62

Figure 22: Total taxa and Microtox EC50 values for Stations 1 and 2		65

Figure 23: Diversity (H’) and Microtox EC50 values for Stations 1 and 2	65

Figure 24: Mixtures by percent weight vs. Mastersizer readings by percent

volume for the mechanically and manually dry sieved samples		69

Figure 25: Effect of particle size on Microtox EC50 values			72

Figure 26: Effect of sample storage time on EC50 values			74

Figure 27: Total polychaete taxa at every station				108

Figure 28: Sheldon’s evenness index at every station				109

List of Tables

Table 1: Summary of the two metrics calculated at Station 1			44

Table 2: Summary of the two metrics calculated at Station 2			44

Table 3: Summary of the two metrics calculated at Station 3			45

Table 4: R2 values for regressions performed for total taxa

with water quality parameters						50

Table 5: R2 values for regressions performed for diversity (H’)

with water quality parameters						51

Table 6: R2 values for regressions performed for all metrics with

Microtox results								63

Table 7: Benthic species collected in Cow Bayou for the August 30,

1996 field trip								86

Table 8: Benthic species collected in Cow Bayou for the September 10,

1996 field trip								87

Table 9: Benthic species collected in Cow Bayou for the October 4,

1996 field trip								88

Table 10: Benthic species collected in Cow Bayou for the December 18,

1996 field trip								89

Table 11: Benthic species collected in Cow Bayou for the January 24,

1997 field trip								90

Table 12: Benthic species collected in Cow Bayou for the February 14,

1997 field trip								91

Table 13: Benthic species collected in Cow Bayou for the March 7,

1997 field trip								92

Table 14: Benthic species collected in Cow Bayou for the March 19,

1997 field trip								93

Table 15: Benthic species collected in Cow Bayou for the April 18,

1997 field trip								94

Table 16: Benthic species collected in Cow Bayou for the May 13,

1997 field trip								95

Table 17: Benthic species collected in Cow Bayou for the June 6,

1997 field trip								96

Table 18: Benthic species collected in Cow Bayou for the July 10,

1997 field trip								97

Table 19: Benthic species collected in Cow Bayou for the July 31,

1997 field trip								98

Table 20: Benthic species collected in Cow Bayou for the September 5,

1997 field trip								99

Table 21: Benthic species collected in Cow Bayou for the October 16,

1997 field trip								100

Table 22: Benthic species collected in Cow Bayou for the December 5,

1997 field trip								101

Table 23: EC50 values for all sediment samples collected from Cow Bayou	102

Table 24: EC50 values for the soil mixtures tested in the experimental

procedure developed to recognize the effects of particle sizes on

the basic solid phase test							103

Table 25: Percent by volume of particles smaller than 75 m in soil mixtures

created after a manual and a mechanical sieving process			104

Table 26: Percent by volume of particles smaller than 75 m in all sediment

samples collected in Cow Bayou						105

Table 27: Field and laboratory measurements of water quality in

Cow Bayou									106

Table 28: Total monthly precipitation (in.) data from the National Climatic

Data Center (NCDC) for the Houston Intercontinental and Galveston

stations									110

List of Equations

Equation 1: Shannon - Wiener (H’) diversity index				42

Equation 2: Standard deviation							43

Equation 3: Coefficient of linear regression (R2)					50

Equation 4: Sheldon’s evenness index						108