Chapter 1  Introduction

- Research Background

This research is part of the project funded by Texas Department of Transportation, titled “Evaluation of the Impacts, Performance, and Costs of Storm Water Pollution Prevention Plans (SW3P) as Applied to Highway Construction Activities”.

The Clean Water Act (CWA) regulates stormwater discharge from construction sites that disturb 5 or more acres (this value may change to 1 or more acres). General permit requirements that apply to construction activities were published in the Federal register, Volume 57, Number 175 on Wednesday, September 9, 1992.

Permittees must provide a site description,  identify sources of contaminants that will affect stormwater, design appropriate measures to reduce pollutants in stormwater discharges, and implement these measures. The appropriate measures are further divided into four classes: erosion and sediment control, stabilization practices, structural practices, and stormwater management. Collectively the site description and accompanying measures are known as the facility Storm Water Pollution Prevention Plan (SW3P). The focus of SW3P is on erosion control and prevention of suspended solids leaving a construction site.

The permit contains no specific performance measures for construction activities, but states that "EPA anticipates that stormwater management will be able to provide for the removal of at least 80% of the total suspended solids (TSS)." In terms of reducing solids load to a receiving stream, the emphasis on solids control is well supported by past studies. In an early study of TSS as a function of land use in Virginia, highway construction areas, varying from less that 1% to more than 10% of the basin contributed 85% of the sediment load. The sediment yield of the highway construction area was 10 times that of cultivated land, 200 times that for grassland, and 2000 times that of forest land. (Vice, 1969). Other investigators found that runoff from highway construction sites causes a temporary degradation of stream water quality with respect to turbidity and suspended solids (Hainly, 1980; Embler and Fletcher, 1981; Helm, 1978).

The goals of this project are to gather, analyze, and interpret existing data to infer the effect of sedimentation on receiving water ecosystems, infer the effects of SW3P on sediment transfer, determine the types and quantities of contaminants associated with highway construction activities, develop useful predictive tools to be used for cost/benefit analysis, and document all activities so that TXDOT can maintain, update and operate database and models as new information becomes available.

- Rainfall-Runoff-Solids Generation Model

One tool under development is a rainfall-runoff-solids generation model to calculate the mass loadings to receiving stream, and identify the effectiveness of SW3P techniques, especially temporary sediment controls (TSC). The model considers two components: detachment and transport by rainfall and detachment and transport by overland flow. The rainfall induced surface erosion, in general, is a power function of rainfall intensity. The sediments scoured in the overland flow are also assumed to be a power function of the overland flow rate. During a runoff producing storm, the user will be able to predict sediment mobilization and the effects of manmade controls on sediment transport using these modeling tools.

This thesis is the very first step of the model, that is to design a rainfall model which can generate 15-minute rainfall sequence for a highway construction site, which will be used in runoff and solids transport model.

1.3 The Organization of This Thesis

This thesis begins by considering the background of the research. The second chapter reviews the literature of rainfall models and concludes that	there is no appropriate rainfall generation model for highway engineers to evaluate effect of SW3P on highway construction sites. Chapter 3 clearly states the problem to be addressed in this thesis.

Chapter 4 presents the synthetic rainfall generation (SRG) model and bootstrapping method development procedure. Model application and limitations are illustrated in Chapter 5. Three assumed highway construction sites, near Austin, Houston and Lubbock, are used to perform the model and statistical test. The effect of number of construction days and number of repeats of SRG model on evaluating mean value of precipitation from statistically generated 15-minute model rainfall sequences is test. Fifteen-minute rainfall sequences from SRG model and bootstrapping method is tested for their likeness and compared for their sediment production.

Chapter 6 presents the conclusions and limitations. Finally Visual Basic and FORTRAN programs source code for the SRG model are attached as appendices as well.

Chapter 2  Literature Review

A rainfall sequence is a type of time series where rainfall depths are recorded in time. The sequence is unidimensional, hydrologic, stochastic process (Yevjevich, 1972). A stochastic process is defined as a mathematical abstraction of a process that can be  characterized by statistical properties. The stochastic process is considered as being mathematically described through three steps: a) distribution functions are postulated, b)  parameters of distribution functions are estimated, c) proper statistical tests of hypotheses are performed. The number and kinds of stochastic models applicable in rainfall generation are nearly unlimited.

Haan (1977) stated that synthetic rainfall from a model that is based solely on historical record of rainfall is but one realization of a stochastic time series and the future realizations will resemble the historical record only in a statistical sense. Stochastic rainfall is neither historical rainfall nor predictions of future rainfall, but a representative of possible future rainfall in a statistical sense.

Pattson (1964) states that stochastic rainfall model implies a repetition in the future of the historical events and in some cases that these future occurrences will take place in the same order as that which has occurred historically. This assumption becomes more valid and the estimates are improved as the length of the historical record is increased.

Haan (1977) describes a purely random stochastic model, which the rain and no-rain events are assumed to occur at discrete times with constant time interval. The events at any time are independent of the events at any other time. The time between storms might be modeled as an independent Poisson process and the amount of rain might be modeled as a gamma variable.

Rodriguez-Iturbe et al. (1984) used a compound Poisson model to analyze the parameter estimation problems for the aggregation of rainfall at hourly and daily scales. This model considered rainfall events in the form of instantaneous burst in time. The occurrence times of the bursts are modeled by a Poisson process. Then to each burst occurrence time an independent rainfall amount is attached. This results in a compound Poisson process for the rainfall amounts in time.

Chang et al. (1984) proposed various models from discrete autoregressive moving average (DARMA) family and applied these models to occurrence of daily rainfall data of Indiana. The most popular approach of these approaches is to consider the precipitation occurrence process to be described by a finite state (typically a value of 2, a day is wet W or dry D) Markov Chain (MC) with seasonally (or time-varying) transition probabilities.

The transition probabilities from transitions (i.e., WW, WD, DW, DD) between the two states (W or D) are estimated directly from the data through a counting process. If the probability law that governs the future development of the process at some point depends only on that point and not on the prior evolution of the process, such a process is called a first-order Markov chain. If, however, the probability law depends on the current state and also on the immediately preceding state then it is called a second-order Markov chain. Chains of Nth-order are defined in the same way. They concluded that the DARMA family of models can preserve various desirable statistical properties of the daily rainfall process.

Roldan and Woolhiser (1982) proposed a wet-dry spell approach for daily precipitation occurrence. This approach is called the alternating renewal model (ARM). No transition to the same state is possible for this approach. An advantage of this approach is that it allows direct consideration of a composite precipitation event, rather than its discontinuous truncation into arbitrary daily segments. Spell length is modeled by a geometric or a negative binomial distribution.

Slade (1936) was the first to fit a continuous probability distribution to rainfall data. He made use of a logarithmic transformation of the normal to fit the annual rainfall amounts.

Skees and Shenton (1974) noted that annual and monthly rainfall were successfully  modeled as random variables with gamma, normal, and logarithmic normal distributions. For shorter intervals (weeks, days, hours), satisfactory distributions were more difficult to obtain.

The S-curve is used in hydrology to model an infinite sequential series of fixed duration storms, which is a step response process (Bras, 1990). This curve is the infinite summation of unit amount of the given duration. The S-curve is obtained by adding corresponding unit hydrograph, each lagged by one unit time.

Dempster (1993) states that a time course fitting procedure can be implemented by defining a suitable step response function for single channel currents. A channel opening can be represented by the sum of an opening step S(t, t0) and a closing step represented by an inverted form (1 - S(t, t0+t)), where S is the amount, t is time, t0 is starting time, t is delay time. The equation of this form is called a convolution equation, and parameters in this equation can be evaluated by optimization method and user-defined criteria.

McCuen (1986) described three types of parameters used to define the distribution of time series as location, scale and shape parameters (i.e., parameters of a distribution control the geometric characteristics of the distribution). Location parameter means location of center. Scale parameter indicates various fractiles of the distribution. Shape parameter is geometric configuration of a distribution, used to distinguish one from another in a family of distributions.

The WEPP USER Requirement (Foster and Lane, 1987) suggested that the maximum information required to represent a design storm consist of the following: (a) storm amount, (b) average intensity, (c) ratio of peak intensity to average intensity, and (d) time to peak intensity.

Tabios and Salas (1985) examine several commonly used spatial interpolation techniques for point annual precipitation at 29 stations located in North Central continental United States. The techniques considered were the followings: Theissen, polynomial, inverse distance, multiquadratic, Gandin, and Kriging. The comparison is based on the following criteria: the mean and variance of the observed and interpolated annual precipitation, the sum of square errors between the observed and interpolated vales of annual precipitation, the proportion of the variance accounted for by the interpolator, the coefficient of determination between the observed and interpolated values and the standard deviation of the error of interpolation.

The result from this study shows that the optimal-interpolation and Kriging techniques are the best among all techniques analyzed considering various performance criteria used by the study. The inverse distance method is not as good as Kriging and optimal-interpolation techniques, but  is better than the Theissen and multiquadratic methods.

There exist many rainfall models, but very rarely is the hydrologist interested in rainfall as an isolated phenomenon. Evaporation, transportation and the antecedent moisture are always considered in a model. Also considered are pertinent characteristics of the basin itself, such as its geology, topography etc. For example the Stanford Watershed Model (Pattison, A, 1964) and the USDA WEPP model (Foster and Lane, 1987) consider all these features.

From the literature reviewed, one concludes that there are many stochastic models based upon historical data to generate a rainfall sequence, but there is no rainfall generation model that meets the requirement for this project: generate 15-minute  sequences, economical data storage so the entire state of Texas can be stored, and easy to use for highway engineers (user-friendly).

The subject of this thesis is the design of an easy-to-use synthetic rainfall generation model for a highway engineer to generate 15-minute rainfall sequences which can preserve total volume of precipitation and some characteristics of intensity of historical data.

Chapter 3 Problem Statement

EPA requires a Storm Water Pollution Prevention Plan (SW3P) to be in effect for highway construction activities. SW3P focus on erosion control and prevention of suspended sediments leaving construction sites.

Sediments produced by storms from highway construction activities effect water body ecosystems by both its sedimentation and as a medium of transporting contaminants.

Simplified models to infer effects of sedimentation and SW3Ps on sediment transfer are desired. The first step of these models is a simulation model to relate rainfall to runoff and total suspended solids generation, which allows a highway engineer to evaluate the planned temporary sediment controls (TSC) that may be part of the Storm Water Pollution Prevention Plan (SW3P).

Many stochastic models exist, but most of them use rainfall data with time space  equal or larger than 1 hour, involve information other than just rainfall, and are not easy to use. Furthermore there is no specific synthetic rainfall generation model for highway construction activities in Texas.

The purpose of  this synthetic rainfall model is to generate rainfall forecasts (in a probabilistic sense) for the rainfall-runoff-solids generation model. This model uses historical daily and 15-minute rainfall data to predict the precipitation during a highway construction period in a probabilistic sense. Some features of this model will be the following:  generation of 15-minute sequences for the whole proposed construction period, economical data storage so the parameter for the entire state of Texas can be stored, and easy-to-use information for highway engineers (user-friendly).

Occurrence of rainfall events in each station is modeled by a sequence of days of  zero rain and nonzero rain properties through a statistical distribution function. Each rainday is modeled by a step response function focusing on the volume, intensity, duration and overall shape of precipitation. A final rainfall sequence at highway construction site is  generated from three nearby stations by spatial interpolation. A user-friendly Visual Basic Interface for SRG model was generated. Model tests and hypothesis tests are performed to check if this model is valid and what the suggested procedures are to use this model.

Chapter 4 Model Development

The SRG model development was divided into three steps: Geographic Information System (GIS) mapping of rainfall  stations in Texas and development of a distance finding procedure, analysis and reduction of historical records into a database, and design of an interface that allows a user to access the data base.

The first step was the construction of a GIS map for rainfall stations in Texas. One hundred and nine out of 206 stations in Texas were selected to build a network of rainfall stations so that there are always some nearby stations around any possible highway construction site in Texas. The distance between the highway construction site and the rainfall stations can be found from this map , and this distance information is used in the SRG model to combine synthetic 15-minute rainfall sequences for each station to generate a 15-minute rainfall sequence for the highway construction site by spatial interpolation.

The second step was daily and 15 minute rainfall data analysis and database construction. Actual daily rainfall data files were analyzed to create empirical cumulative arrival time function (ECATF) parameter data files. Actual 15-minute data files were analyzed to create rainday parameter data files for use in a convolution equation. These parameter data are one option in the SRG model to generate synthetic 15-minute rainfall sequences for each station. Another option in the model is to resample the original data (bootstrap).

The third step was to develop a Visual Basic interface to allow the user to access the data files and generate rainfall sequences.

4.1 Construction of GIS Map and Distance Finding

In order to easily select three nearby stations and find the distances between stations and highway construction site, a Geographic Information System (GIS) (Strategic Mapping Inc., 1994) computer program was used to construct a map of Texas so that both rainfall stations and highway construction site can be viewed in the map and the distances between them can be calculated automatically.

The map was constructed by selecting 109 out of 206 stations from the original rainfall database using the following criteria:

1) All stations around major cities are available on the map.

2) Stations far away from cities are selected so that there are always at least three  stations within 100 miles around any possible HCS site in Texas.

3) Choose existing stations with a longer historical record.

The longitude and latitude data of rainfall stations are extracted from the original database and are formatted so that rainfall stations can be displayed on the GIS map. Figure 4-1 shows the GIS map for the 109 selected rainfall stations in Texas. On this map, counties, major cities, and interstate highways are displayed so that a location is easy to find. Each rainfall station is represented by a cross flag. A user can zoom in on a map section to see more detail as needed.

Distances between rainfall stations and highway construction sites are computed in the GIS using the calculate column algorithm (Appendix A2). These distances are used in subsequent calculations.

A spatial interpolation method is used to generate a 15-minute time series for the HCS by averaging 15-minute time series from the three nearby stations using an inverse distance algorithm,

ZH.C.S.=i=1nZi(di)2i=1n1(di)2,	(4-1)

where ZHCS  is precipitation at the HCS, Zi is precipitation at each rainfall station, and di is distance from the HCS to each rainfall station.

The inverse distance method belongs to a family of weighting techniques, that instead of simple averaging of the rainfall data applies more weight to the rainfall data that from the stations closer to the highway construction site.

4.2 Daily and 15-Minute Rainfall Data Reduction

The historical daily and 15-minute rainfall data were obtained from EarthInfo, Inc. (EarthInfo, Inc., 1995). The length of records varied from 10 to 50 years at the different stations.

The procedure of data treatment is depicted in Figure 4-2. Actual daily and 15-minute data (the upper left box) are first extracted from original database and  then formatted by manually deleting bad records and changing the data files into the proper format (the upper right 3-D box). For both daily and 15-minute rainfall data, there were up to 10%  missing or improperly formatted records. These “bad” records were deleted manually so that the data files are readable by FORTRAN programs. Finally parameter generation (the lower middle 3-D box) programs are used to generate daily and 15-minute parameters (model data) represented by the lower right box.

Figure 4-2  Procedure of Data Treatment

D.R.D.: daily rainfall data; F.R.D.: 15-minute rainfall data

A typical portion of an original daily rainfall record set for one particular year is illustrated in Figure 4-3. The separation mark between records is “|”. The first record is the station ID (“428”), and the second record is the record year (“1942”). This particular record set has too many missing records (indicated by consecutive “||”), so it is entirely deleted from the database.

428|1942|||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||0.07|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.20|0.00|0.00|0.00|1.89|0.00|0.12|0.00|2.60|2.32|0.49|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.34|0.00|0.65|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.01|0.01|2.15|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|1.61|0.00|0.20|0.71|0.01|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.01|0.00|0.01|0.42|0.00|0.00|0.00|0.00|0.74|0.01|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.02|1.21|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.02|0.02|0.00|0.09|0.06|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|1.21|0.00|0.00|0.00|0.02|0.11|0.00|0.00|0.00|0.00|0.00|

Figure 4-3  A Typical “Bad” Original Daily Data Record Set

Next, data sets that are completed, such as the example in Figure 4-4, are reformatted to remove the separation marks, and to have a four-character station ID. Figure 4-5 illustrates how a completed, reformatted data set appears.

A similar procedure is applied to the 15-minute rainfall records in these data files. Figure 4-6 shows a typical “bad” data set. Separation marks are “||” for 96 rainfall records, and “|” for other supplementary records. The first record is the state ID (“TX” for Texas), the second record is the station ID (“8531”), and the fifth record is the date (“01/02/1984). The mark “---“ indicate a missing records.  The character “A” after the number (“1.07”) indicates that 1.07 is an accumulative rainfall record from an unknown time  interval  (not  15-minute  interval).  The  character  “I”  after  the  number  (“1.07”)

428|1943|0.00|0.00|0.00|0.00|0.03|0.32|0.00|0.00|0.00|0.00|0.05|0.38|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.02|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.15|0.01|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.29|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.01|0.00|0.00|0.00|0.00|0.00|0.00|0.01|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|2.52|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|2.65|0.00|0.00|0.03|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.43|0.00|0.00|0.00|2.05|0.00|0.00|0.00|0.00|0.01|0.00|0.00|0.00|0.00|0.00|0.00|0.49|0.78|0.17|0.00|0.00|0.00|0.23|0.63|0.59|0.00|0.00|0.00|0.01|0.28|0.17|0.03|0.00|0.00|0.00|0.00|0.00|0.31|0.06|0.01|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.05|0.00|0.00|0.00|0.13|0.22|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.01|1.17|0.45|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.04|0.01|0.01|2.17|0.05|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.16|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.76|0.00|0.18|0.16|0.01|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.11|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.37|1.07|1.00|0.15|0.23|0.03|0.00|0.04|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.06|0.23|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.81|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.10|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.33|0.39|0.10|0.00|0.00|0.00|0.00|0.00|0.00|0.11|0.00|0.25|0.03|0.00|0.08|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.00|0.11|0.21|0.20|0.00|0.00|0.42|0.00|0.00|0.00|0.01|

Figure 4-4  A Typical Acceptable Original Daily Data Set

0428   1943   0.00   0.00   0.00   0.00   0.03   0.32   0.00   0.00   0.00   0.00   0.05   0.38   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.02   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.15   0.01   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.29   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.01   0.00   0.00   0.00   0.00   0.00   0.00   0.01   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   2.52   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   2.65   0.00   0.00   0.03   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.43   0.00   0.00   0.00   2.05   0.00   0.00   0.00   0.00   0.01   0.00   0.00   0.00   0.00   0.00   0.00   0.49   0.78   0.17   0.00   0.00   0.00   0.23   0.63   0.59   0.00   0.00   0.00   0.01   0.28   0.17   0.03   0.00   0.00   0.00   0.00   0.00   0.31   0.06   0.01   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.05   0.00   0.00   0.00   0.13   0.22   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.01   1.17   0.45   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.04   0.01   0.01   2.17   0.05   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.16   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.76   0.00   0.18   0.16   0.01   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.11   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.37   1.07   1.00   0.15   0.23   0.03   0.00   0.04   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.06   0.23   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.81   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.10   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.33   0.39   0.10   0.00   0.00   0.00   0.00   0.00   0.00   0.11   0.00   0.25   0.03   0.00   0.08   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.00   0.11   0.21   0.20   0.00   0.00   0.42   0.00   0.00   0.00   0.01

Figure 4-5  A Typical Reformatted Daily Data Set

indicates that 1.07 is an accumulative rainfall record for the whole day. This particular day has too many missing data, so it is deleted entirely from the database.

TX|8531|6|in|01/09/1984|---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||---||1.07|A|.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||1.07|I|

Figure 4-6  A Typical “Bad” Original 15-min. Data Record Set

Figure 4-7 illustrates an acceptable day for data from rainfall station 8531 near Austin.

TX|8531|6|in|05/19/1984|.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||0.01||0.01||0.01||0.01||0.02||0.02||0.02||0.02||0.02||0.04||0.10||0.09||.||.||0.01||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||.||0.38||

Figure 4-7  A Typical Acceptable Original 15-min. Data Set

This data set is reformatted by replacing the separation marks “||” and “|” with two spaces and replacing “.” (an abbreviation for 0.00) with “0.00”. Figure 4-8 illustrates the result of reformatting.

TX  8531  6  in  05/19/1984  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.01  0.01  0.01  0.01  0.02  0.02  0.02  0.02  0.02  0.04  0.10  0.09  0.00  0.00  0.01  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.38

Figure 4-8  A Typical Reformatted 15-min. Data Set

After the data are reformatted, the database is analyzed to construct rainfall data parameters and 15-minute data parameters.

The concept of arrival time and an arrival time series is shown in Figure 4-9. A nonzero rainday (a day when rain occurs) is represented by a shaded box, and a zero rainday is represented by a plain box. The arrival time is defined as the number of consecutive zero raindays, and is equal to zero for a nonzero rainday. An arrival time series is a sequence of arrival times. For the daily rainfall sequence shown in Figure 4-9, the arrival time series is 0, 1, 0, 0, 3, 0, 2, 0.

Analysis of arrival times from historical real daily rainfall data for Texas indicates that arrival times appear to be Poisson distributed (Lapin, 1973). In Figure 4-10, the y-axis is the relative frequency f(t) and the x-axis is arrival time t in days. The relationship between relative frequency and arrival time shown in Figure 4-6 can be described by

Figure 4-9 Definition of Arrival Time Series

f(t)=e−λt,	(4-2)

where f(t) is relative frequency, l is the mean rate of arrival time, and t is arrival time.

Another way to represent this information is by a cumulaltive distribution approach as shown in Figure 4-11. The y-axis is the cumulative frequency and the x-axis is arrival time t in days. This type of distribution is described by a cumulative arrival time function, which is

P=1−e−λt,	(4-3)

t= -1λ ln(1-P),	(4-4)

where P is the cumulative probability from 0 - 1; t is arrival time in days; and l is the mean rate of arrival time, 1/day. One way to interpret this distribution is in terms  of exceedance probabilities. Figure 4-11 shows the probability that the arrival time of the

Figure 4-10  Relative Frequency Function of Arrival Time

Figure 4-11  Empirical Cumulative Arrival Time Function

next nonzero rainday being less than or equal to 70 is nearly 1 (very likely).

These cumulative arrival-time function (ECATF) parameters are generated for each station by the ECATFPG model (Appendix A4). That is, a set of arrival times with a set of corresponding numbers between 0 and 1 is obtained.

To sample from these distributions a randomly generated number between 0 and 1 is associated with a corresponding arrival time. For example, if the P value is 0.6, then the arrival time is 6 days. This inverse transformation is the idea used in the random arrival time module in the SRG model to generate arrival times sequences. The simulated arrival time series is generated by adding zero raindays and nonzero raindays so that total number of days in the arrival-time sequence is equal to the number of construction days.

The mean rate of arrival time is the reciprocal of mean arrival time. Therefore a larger mean rate of arrival time is equivalent to a short arrival time, or more frequent rain events. Figure 4-12 shows the calculated mean rates of arrival times for the 109 rainfall stations in Texas.  The mean rates of arrival times are larger in the east of  Texas as compared to that of western Texas. In another words, there are fewer days between rain events in the eastern part of Texas than that in the western part of Texas. The shortest mean arrival time is 3.5 days near the Texas-Louisiana border, and the longest is 14.2 days near Del Rio on the Texas-Mexico border.

Individual behavior on nonzero rain days are modeled using a four-parameter equation (Bras, 1990 and Dampster, 1993) that can roughly mimic overall shape, rain duration,  and  total  precipitation of  historical rain events on a daily basis.  An exact fit is not expected. The equation is based on the concept of convolution where individual unit inputs are integrated to produce an overall response. The S-curve method in hydrology is an example of convolution applied to runoff hydrographs.

Figure 4-13 shows the concept of convolution. Figure 4-13(a) shows the starting of a rain event as a diffuse step response function (eqn. (4-5)) with an ultimate value of R0, tp is the time for rain event to reach this value. Figure 4-13(b) shows the ending of a rain event by as the inverse response of Figure 4-13(a) with a lag time of t (eqn. (4-6)). Figure 4-13(c) is synthesized from the sum of Figure 4-13(a) and 4-13(b).

The symbols r1(t) and r2(t) represent two step-response function:

r1(t)=R04pSdtexp(−(tp−t)24Sdt),	(4-5)

r2(t) = - r1(t) with a lag time of t.	(4-6)

Let

F1(t)=r1(t)dt,	(4-7)

F2(t)=r2(t)dt,	(4-8)

Then, we get

R(t)=F1(t)+F2(t).	(4-9)

Figure	4-13  Graphic Representation of Convolution Equation

To find F1(t), we integrate

F1(t)=r(t)dt=R04pSdtexp[−(tp−t)24Sdt]dt	(4-10)

Let

u=tp−t2Sdt,

t=Sdu2+tp−Sdu,	(4-11)

dtt=2[SduSdu2+tp−Sd]du.	(4-12)

Then, we have

F1(t)=R02pSd´exp(−u2)´2´(SduSdu2+tp−Sd)du, 	(4-13)

=-R022pexp(−u2)du+R0SdpSduSdu2+tpexp(−u2)du,

=R02erfc[tp−t2Sdt]+R0pSdexp(−u2)SdSdd(u2Sd+tp),	(4-14)

Let

w=u2Sd+tpSd.	(4-15)

Then, we have

F1(t)=R02erfc[tp−t2Sdt]+R0pexp(−w2+tpSd)dw,	(4-15)

=R02erfc[tp−t2Sdt]−R02exp(tpSd)erfc[tp+t2Sdt].	(4-16)

Similarly, F2(t) is

F2(t)=-R02[erfc(tp−(t−t)2Sd(t−t))−exp(tpSd)erfc(tp+(t−t)2Sd(t−t))].	(4-17)

From eqn. (4-9), we have

R(t) = F1(t) + F2(t).

Then, R(t) is equal to

R(t)=12R0´[erfc(tp−t2Sdt)−exp(tpSd)erfc(tp+t2Sdt)]−[erfc(tp−(t−t)2Sd(t−t))−exp(tpSd)erfc(tp+(t−t)2Sd(t−t))],	(4-18)

where R is the predicted precipitation (in.), t is the elapsed time (15-minute on a daily basis, or from 15 minutes to 1440 minutes) since the beginning of the rain day, and four rainday parameters are the following: R0, peak rainfall amount (in.); tp, historical time to peak; t, rain duration; and Rd, rain dispersion coefficient.

The four rainday parameters in this model are found by minimizing the sum of squared differences between the observed and modeled rainfall values. This minimization is performed using a constrained Quasi-Newton method (Press, 1986) with the criterion that total volume of rainfall is preserved in the model. Figure 4-14 shows the results of this model approach for three different cases. The y-axis is the amount of precipitation in inches, and the x-axis is time in minutes. As the complexity of the rainfall pattern increases in one day (From Figure 4-14(a) to 4-14(c)), the model loses the ability to simulate the real rainfall intensity, but total volume, overall shape, and rain duration are preserved. This rainday model to calculate 15-minute rainfall data from four rainday parameters is the parametric based option for generating 15-minute rainfall data in the SRG model.

Figure 	4-15 shows that one benefit from data reduction is the space needed to store data in a computer is significantly reduced. For daily data, there is a reduction of the file size by a factor of  30.  For 15-minute data,  there is a reduction of the file size by a factor of

Figure 4-14(a)  15-minute Model vs. Real Data

Figure 4-14(b)  15-minute Model vs. Real Data

Figure 4-14(c)  15-minute Model vs. Real Data

10. So after the parametric analysis, daily data files for all stations in Texas can be stored on one floppy disk. Similarly, 15-minute data all for stations around Houston can be stored on one high-density floppy disk.

Figure 4-15  Comparison of Storage Space for Model and Real Data

D.R.D.: daily rainfall data; F.R.D.: 15-minute rainfall data

4.3. SRG Model Interface Design

The SRG model operates on the data developed in the previous two steps. Figure 4-12 shows the  flow chart of SRG model to use the 15-minute model data.

The user supplied data required for the SRG model is entered through the interface (the upper central box). These data are: number of construction days, distances from construction sites to selected stations, and station identification numbers of those stations. Station ID numbers are used by the SRG model to find corresponding daily and 15-minute parameter data files from the database. The model accesses these data to construct a rainfall sequence for the highway construction site being studied..

The computational sequence in the SRG model is (Figure 4-16):

- The main module (the central upper 3-D box) will first call the random arrival time (RAT) module (the middle left 3-D box). The RAT module will read the ECATF parameters from the corresponding ECATF parameter data file (the upper left box), create a series of random numbers between zero and one according to the number of construction days; and then, generate an arrival time series (the lower left box) for each station identified in the input.

2) For those days in the generated arrival time series that are identified as raindays, the main module will call the random rainday (RRD) module (the middle right 3-D box). The RRD module will randomly select four rainday parameters from the 15-minute parameter data files (the upper right box), and produce 15-minute rainday sequence (the lower right box) from the convolution equation.

- Finally, the main module will call the inverse distance (DIST) module (the central lower 3-D box) to generate the 15-minute rainfall sequence (the lower central box) for the construction site by combining the 15-minute rainfall sequences from the three used identified stations.

Figure 4-16  Flow Chart of the SRG Model

Most of the computational functionality and database information in the SRG model is coded in FORTRAN. A Visual Basic Interface is constructed to simplify the use of the SRG model. Figure 4-17 shows the relationship among user, interface and the SRG model. Instead of creating an input file and running the SRG model directly, the user (the left box) inputs the required site specific information and executes the SRG model calculations (the right box) through several labeled information boxes and command buttons on the interface (the central box).

Figure 4-18 shows the structure of the Visual Basic Interface. The main module (the left box)  is  used  for  information input and program control.  From the main module, three

Figure 4-17  Overall Structure of the SRG Model

functions are available. The input file creation function (the right upper box) is used to create the input file for the SRG model calculations. The help function (the right central box) is available to assist the model user. The graphics function (the right lower box) is used to generate  a plot of 15-minute rainfall sequence for the user to inspect.

Figure 4-18  Flow Chart of the SRG Model Interface

Figure 4-19 shows the SRG model Interface. A user enters required data in the nine boxes. The interface program checks the validity of input information, and performs calculations based upon the user’s choice of the parametric or the bootstrapping method. A user can also generate a plot of 15-minute rainfall sequence and get help using the SRG model.

Figure 4-20 shows a typical 15-minute rainfall sequence from the SRG model. The number of construction days is equal to 20. The y-axis is the amount of precipitation in inches, and the x-axis is the time in days. From these rainfall sequence diagrams, possible occurrence of rain events, rain intensity and volume of rain precipitation can be estimated.

Figure 4-20  Rainfall Sequence from the SRG model

Chapter 5 Bootstrapping Method

Bootstrapping is a generic name used to describe the process of resampling historical data. It differs from the SRG model (a parametric approach) in that it models the characteristics of a population strictly from the sample at hand, rather than by making assumptions (possibly unrealistic) about the population, distribution equations, and descriptive parameters.

At the beginning of this research, the primary concern about using bootstrapping was how to effectively resample from a huge data set. In particular, there were 10 to 50 years of historical daily and 15-minute rainfall records from the 109 rainfall stations, with a total number of rainfall data points exceeding 10 million. As many as 10% of the records were missing or irregularly formatted. The parametric SRG model was created to learn how to manage a large data set and treat missing or bad records, and change them to useable format. Once the SRG model was constructed, the bootstrapping method to resample real data was designed.

Figures 5-1(a-c) illustrate that the model preserves the overall duration, overall shape and total amount of precipitation, and does not preserve peak intensity. Alternatively, if preservation of peak intensity is emphasized, then it is impossible for any smooth and continuous curve to preserve overall duration, overall shape, and total amount of precipitation. Jilani (1997) and Liu (1997) suggest that intensity is more important than total amount of precipitation in predicting erosion. To preserve precipitation intensity

Figure 5-1(a)  15-minute Model vs. Real Data

Figure 5-1(b)  15-minute Model vs. Real Data

Figure 5-1(c)  15-minute Model vs. Real Data

information, the bootstrapping method was designed and added to the model as an alternative way to generate  rainfall sequences.

The bootstrapping method in this thesis selects raindays during a construction using the empirical cumulative arrival time distributions in the first step. Then the rainday behavior is modeled by selecting a historical day from the database and using the rainfall data for that day. Seasonal effects, wet/dry year effects are ignored in this method.

The computational sequence for the bootstrapping method is illustrated in Figure 5-2. The  procedures  is nearly identical to  the  parametric  SRG model except for how the

rainday 15-minute precipitation values are computed.

Figure 5-2  Flow Chart of the Bootstrapping Method

The sequence of computations is

1) The main module (the central upper 3-D box) will first call the random arrival time (RAT) module (the middle left 3-D box). For each station, the RAT module reads the ECATF parameters from the corresponding ECATF parameter data file (the upper left box). Then the RAT module generates a random number between 0 and 1, and identifies the corresponding arrival time according to ECATF parameter. The RAT module repeats these steps in order to form an arrival time series (the lower left box) in which the total number of raindays and zero raindays is equal to the number of construction days. Three simulated arrival time series (one for each station) are sent back to the main module.

2) The main module analyzes the simulated arrival time series created from the RAT module. For each rainday in the generated arrival time series, the main module will call the random rainday (RRD) module (the middle right 3-D box). The RRD module will generate a random number between 1 and the total number of 15-minute data sets for a particular station (One data set stores 96 15-minute rainfall data points for one rainday.). The module then selects the corresponding rainday from 15-minute data files (the upper right box), and reads 96 rainfall data values for that rainday (the lower right box). These  rainfall data values for each rainday are returned to the main module. For each zero rainday, the main module will assign 96 zero values. The main module joins these zero and nonzero precipitation data points together on a 15-minute basis to form three rainfall sequences (one for each station) according to the order of generated arrival time series from the RAT module.

- Finally, the main module will call the inverse distance (DIST) module (the central lower 3-D box). The DIST module will take three real rainfall data values of the same time from the three different sequences (one rainfall data value from each sequence) and calculate the rainfall data value of highway construction site from these three data values by the inverse distance algorithm (eqn. (4-1)). Then  the DIST module will join the generated rainfall data values for highway construction site one by one and form 15-minute rainfall sequence (the lower central box) for the construction site.

To illustrate the procedure of the bootstrapping method with an example, the assumed highway construction site HCS_1 near Austin is used (Figure 5-3). The required information to use the bootstrapping method is entered through the Visual  Basic Interface (Figure 4-19): the number of construction days is set to be equal to 20; three stations used to perform rainfall generation for HCS_1 are 0428, 8531, and 9815; distances from HCS_1 to these three stations are 15, 18, and 25 miles, respectively.

Figure 5-3  GIS Map Showing HCS_1 near Austin

Figure 5-4 shows a 15-minute rainfall sequence for the assumed highway construction site HCS_1. The y-axis is the amount of precipitation in inches, and the x-axis is the time in days.

In principle this bootstrapping method will preserve historical rainfall intensity. The pattern of Figure 5-4 is identical to that of Figure 4-20, but the intensities are greater. In fact the values in Figure 4-20 cannot even be measured using standard rain gages.

Figure 5-4  Rainfall Sequence from the Bootstrapping Method

Chapter 6 Model Test

The parametric SRG model presented in Chapter 3 and 4 was tested by asking the following questions: 1) What is the effect of the number of construction days and the number of repeats on SRG model’s mean value of precipitation? 2) Is the parametric model sequence distinguishable from a rainfall sequence generated by the bootstrapping method (real rainfall sequence)? 3) If the answer to the second question is yes, then, will the model and real rainfall sequences produce the same amount of soil loss using the soil production model (Jilani, 1997)?

6.1 Introduction

Three assumed highway construction sites near Austin, Houston and Lubbock are selected to test the SRG model. Table 6-1 shows location information for these three sites. Figures 6-1(a), (b) and (c) show location of these sites (solid triangle enclosed by  solid  circle)  along  with  selected  rainfall  stations  for  15-minute rainfall  sequence

Table 6-1  Site Information Sheet for HCS

| Site Name | Site ID | Latitude | Longitude |
| --- | --- | --- | --- |
| AUSTIN No.1 | HCS_1 | 30.33 | 95.91 |
| HOUSTON No.1 | HCS_2 | 29.83 | 95.50 |
| LUBBOCK No.1 | HCS_3 | 33.83 | 101.17 |

Figure 6-1(a)  GIS Map Showing HCS_1 near Austin

Figure 6-1(b) GIS Map  Showing HCS_2 near Houston

Figure 6-1(c)  GIS Map Showing HCS_3 near Lubbock

generation (cross flag enclosed by dashed circle) on the GIS map. Also the distances from HCS to selected rainfall stations are shown on the map.

To perform a statistical test, two commonly used statistical parameters, mean and variance, will be  used throughout the model test procedure. The mean is the arithmetic average of a set of data, which is defined as

m=xin,					(5-1)

where m is the mean value, xi is the sample data, n is number of data points. The variance is used to define the width (spread) or variability around a central value, and is determined by

s2=(xi−m)2n.				(5-2)

where s2 is the variance, xi is the sample data, m is the mean or mean value, and n is number of data points.

6.2 Model Testing for the Effect on Number of Construction Days and Number of Repeats

The purpose of the first test is to determine whether the number of construction days (NCD) and number of repeated runs (NR) of the SRG model will affect the mean values of precipitation for parametrically generated rainfall sequences. The numbers of construction days used are 10, 20, 30, 60, 90 and 120,  and the number of repeats are 10, 20, 30, 40, 50, and 60.

The assumed highway construction site HCS_1 and three surrounding rainfall stations are  selected for this test (Figure 6-1(a)). For each NCD, a number of model rainfall sequences  are  generated  according to  NR  by  the  SRG  model. For example, if NR

equals to 10, then ten sets of rainfall sequences are generated, or the SRG model is run ten times. Then the average value for each time unit (15-minute) from these rainfall sequences is computed one by one. Then these average values are connected together in the same order in time, forming the parametrically generated rainfall sequence for that NR. Table 6-2 shows the mean and variance of precipitation for the statistically generated model rainfall sequences for different NCD and NR combinations.

Figure 6-2 shows  the  mean values of  precipitation for  the  parametrically  generated model rainfall sequences from different NCD and NR. This figure shows that for values of NCD less than 60 days, there is large fluctuation among mean values for different  numbers  of   repeats.   For  each   proposed  NCD,  the variances  for  the  mean values of precipitation from different NR are calculated and are shown in Table 6-3.  The  variances  for  mean  values  of precipitation decrease steadily with increasing NCD. For any NCD equal to or larger than 60 days, the variances are in the order of 10-10. The reasons for this result are: 1) As sample size increases (NCD becomes larger), the mean of sample approaches the mean of population; 2) Analysis of the arrival times for rainfall events in Texas shows that maximum arrival time is around 60 days. Therefore, to run the SRG model, the NCD equal to or larger than 60 are needed so that all the probabilities of arrival times will be considered in the modeling process.

For values of NCD equal to and larger than 60, the variances of the mean values of precipitation for NR equal to 10, 20, 30, 40, 50 and 60 are calculated and shown in Table 6-4. In  Figure 6-2 and Table 6-4, there exists no clear trend of variance change, and the difference in variances is within three times except for NR equal to 10. Therefore a value of  NR equal to or larger than 20 is suggested to run the SRG model to get the mean value of precipitation for statistically generated rainfall sequences. The total volume of precipitation expected during a construction period can be calculated by multiplying the mean values of precipitation by duration.

Table 6-2  Result from Different NCD and NR

| Number of Construction Days | Number of Data | Number of repeats | Mean (in.) for Model Data | Variance for Model Data | Mean (in.) for Real Data | Variance for Real Data |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 960 | 10 | 6.16E-04 | 1.11E-06 | 4.99E-04 | 4.58E-06 |
|  |  | 20 | 7.98E-04 | 5.41E-07 | 5.29E-04 | 3.17E-06 |
|  |  | 30 | 5.98E-04 | 3.04E-07 | 5.78E-04 | 2.08E-06 |
|  |  | 40 | 7.99E-04 | 2.94E-07 | 7.55E-04 | 2.31E-06 |
|  |  | 50 | 5.41E-04 | 1.54E-07 | 5.06E-04 | 9.35E-07 |
|  |  | 60 | 7.04E-04 | 2.34E-07 | 6.65E-04 | 1.04E-06 |
| 20 | 1920 | 10 | 6.41E-04 | 9.53E-07 | 6.12E-04 | 6.63E-06 |
|  |  | 20 | 6.66E-04 | 4.65E-07 | 6.51E-04 | 3.54E-06 |
|  |  | 30 | 5.41E-04 | 2.57E-07 | 5.32E-04 | 1.96E-06 |
|  |  | 40 | 5.63E-04 | 2.35E-07 | 5.56E-04 | 1.56E-06 |
|  |  | 50 | 5.54E-04 | 1.58E-07 | 5.43E-04 | 1.09E-06 |
|  |  | 60 | 5.69E-04 | 2.41E-07 | 5.35E-04 | 1.27E-06 |
| 30 | 2880 | 10 | 4.50E-04 | 5.48E-07 | 4.11E-04 | 4.97E-06 |
|  |  | 20 | 5.97E-04 | 5.78E-07 | 5.88E-04 | 3.94E-06 |
|  |  | 30 | 6.07E-04 | 4.17E-07 | 5.55E-04 | 2.38E-06 |
|  |  | 40 | 5.89E-04 | 3.08E-07 | 5.55E-04 | 1.76E-06 |
|  |  | 50 | 5.73E-04 | 2.44E-07 | 5.37E-04 | 1.27E-06 |
|  |  | 60 | 6.59E-04 | 2.43E-07 | 6.24E-04 | 1.31E-06 |
| 60 | 5760 | 10 | 6.51E-04 | 1.19E-06 | 6.02E-04 | 5.72E-06 |
|  |  | 20 | 5.92E-04 | 4.91E-07 | 5.54E-04 | 3.17E-06 |
|  |  | 30 | 5.84E-04 | 3.84E-07 | 5.39E-04 | 2.44E-06 |
|  |  | 40 | 5.83E-04 | 2.75E-07 | 5.59E-04 | 1.53E-06 |
|  |  | 50 | 5.95E-04 | 2.66E-07 | 5.49E-04 | 1.46E-06 |
|  |  | 60 | 5.74E-04 | 1.86E-07 | 5.36E-04 | 1.18E-06 |
| 90 | 8640 | 10 | 6.03E-04 | 1.05E-06 | 5.51E-04 | 5.11E-06 |
|  |  | 20 | 5.68E-04 | 4.41E-07 | 5.39E-04 | 2.77E-06 |
|  |  | 30 | 5.81E-04 | 3.56E-07 | 5.49E-04 | 2.15E-06 |
|  |  | 40 | 5.78E-04 | 2.88E-07 | 5.41E-04 | 1.66E-06 |
|  |  | 50 | 5.76E-04 | 1.94E-07 | 5.34E-04 | 1.03E-06 |
|  |  | 60 | 6.04E-04 | 2.61E-07 | 5.77E-04 | 1.53E-06 |
| 120 | 11520 | 10 | 5.88E-07 | 7.67E-04 | 5.02E-04 | 4.72E-06 |
|  |  | 20 | 5.67E-04 | 4.68E-07 | 5.30E-04 | 2.88E-06 |
|  |  | 30 | 6.02E-04 | 3.74E-07 | 5.63E-04 | 2.16E-06 |
|  |  | 40 | 6.02E-04 | 3.07E-07 | 5.67E-04 | 1.70E-06 |
|  |  | 50 | 5.92E-04 | 2.42E-07 | 5.57E-04 | 1.28E-06 |
|  |  | 60 | 5.87E-04 | 2.17E-07 | 5.53E-04 | 1.23E-06 |

Figure 6-2  Mean Value vs. Number of Repeat

Table 6-3  Variance of Mean Values of Precipitation from Each Proposed NCD

| NCD | 10 | 20 | 30 | 60 | 90 | 120 |
| --- | --- | --- | --- | --- | --- | --- |
| Variance of Means | 1.2E-08 | 2.6E-09 | 4.9E-09 | 7.7E-10 | 2.2E-10 | 1.7E-10 |

Table 6-4  Variance of Mean Values from Each Proposed NR

| Number of Repeats | 10 | 20 | 30 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- |
| Variance of Mean Value of Precipitation | 7.2E-10 | 1.3E-10 | 8.6E-11 | 1.1E-10 | 6.9E-11 | 1.5E-10 |

- Model Testing for Model and Real Sequences

The purpose of the second test is to determine is the model rainfall sequence is distinguishable  from the real rainfall sequence generated from the bootstrapping method.

A value of NCD equal to 60 is used as input.

From Table 6-2, the variances of precipitation for the model and real rainfall sequences are quite different, and that is because an exact fit of the 15-minute real and model data on daily basis is not expected. As shown in Figure 4-14, though the model sequence preserves the overall shape and total volume of the real sequence, it produces a continuous and uniform curve for the discontinuous and spike-like real sequence on each day. Therefore the variance, which indicates the width or variability around a central value, is decreased for the model sequence.

An unequal t-test (Blaisdell, 1992) is used to test whether real and model rainfall sequences have the same mean values of precipitation. This test method is used to test whether two populations have the same mean values when their variances are different.

1. Assumptions for the test are as follows:

a) independent random samples,

b) each population has a normal distribution.

2. Hypothesis and test statistics are as follows:

a) The null hypothesis (H0) is

H0: (m1-m2) = 0,	 				(6-3)

where m1 is the mean of precipitation for the model rainfall sequence, and m2 is the mean of precipitation for the real rainfall sequence.

b) The test statistic for H0 is

t=(m1−m2)−0s12n1+s22n2,				(6-4)

where t is the t-test value or t value, s12 is the variance of precipitation for the model sequence, s22 is the variance of precipitation for the real sequence, n1 is the number of data points for the model rainfall sequence, and n2 is the number of data points for the real rainfall sequence.

c) Alternative hypothesis is given by

Ha: (m1-m2) ¹ 0,					(6-5)

where the mean of precipitation for the model rainfall sequence is not the same as that of the real rainfall sequence. The rejection region for H0 is defined as t < - ta/2 or t > ta/2. In other words, when t < - ta/2 or t > ta/2, Ha is accepted, and H0 is rejected.

d) The degrees of freedom of the t-distribution are given by

df=(s12n1+s22n2)2(s12n1)2n1−1+(s22n2)2n2−1,				(6-6)

where df is the value of the degrees of freedom, and is rounded down to the nearest integer to make a more conservative approach.

3. The test procedure is as follows:

a) Significance level is set to be 0.1.

- The t values are calculated.

Table 6-5 shows the values of the t-test and degrees of freedom (df) for nine cases from three assumed highway construction sites. The t values vary from 0.000 to 0.354.

c) Conclusion

From a statistical table (Daniel and Terrell, 1978), for percentiles of the t distribution, ta/2 (t0.05) = 1.645, so t< ta/2.

The null hypothesis is accepted since t = 0.000 to 0.354 does not lie in  the  rejection  region. Thus, with a = 0.1, there is sufficient evidence to conclude that the means of precipitation for the model and real data are equal.

Table 6-5  Statistical Test for Rainfall Sequence

| Site ID | Trial Number | Mean (in.) for Model Data | Variance for Model Data | Mean (in.) for Real Data | Variance for Real Data | t value | df value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HCS_1 | 1 | 9.13E-04 | 1.06E-05 | 9.12E-04 | 4.45E-05 | 0.010 |  | 8355 |
|  | 2 | 6.64E-04 | 1.10E-05 | 6.63E-04 | 9.90E-05 | 0.007 |  | 7023 |
|  | 3 | 3.35E-04 | 2.29E-06 | 3.35E-04 | 1.48E-05 | 0.000 |  | 7500 |
| HCS_2 | 1 | 7.99E-04 | 7.15E-06 | 7.57E-04 | 7.88E-05 | 0.344 |  | 6796 |
|  | 2 | 1.04E-03 | 4.13E-05 | 9.93E-04 | 6.05E-05 | 0.354 |  | 11122 |
|  | 3 | 4.86E-04 | 2.02E-05 | 4.86E-04 | 3.72E-05 | 0.000 |  | 10589 |
| HCS_3 | 1 | 1.12E-04 | 1.59E-06 | 1.12E-04 | 4.31E-06 | 0.000 |  | 9499 |
|  | 2 | 3.18E-04 | 1.27E-05 | 3.18E-04 | 1.97E-05 | 0.000 |  | 11004 |
|  | 3 | 2.75E-04 | 8.49E-06 | 2.74E-04 | 1.61E-05 | 0.015 |  | 10511 |

6.4 Model Testing Using VISIOSED Model

The soil production model VISIOSED designed by Jilani (1997) is used to  test  whether  the soil  production  from  the  model  and  real  rainfall  sequences  are identical.

This VISIOSED model is developed from the Universal Soil Loss Equation (USLE)  and calculates the mass of sediment leaving a highway construction site depending on the rainfall intensity, land slope, type of soil, type of control practice, and type of vegetation cover used to control erosion.

The field constraints entered for this model are as follows:

Temporary sediment controls (TCS): rock filter dam vs. silt fence

Soil type: silty clay/clay with organic contents equal to 0.5%

Type of cover: seeding after 60 days

Length from the point of origin of overland flow to the point where the runoff enters a defined channel: 5280 ft (1 mile)

Average overland slope from the point of origin of overland flow to the point where the runoff enters a defined channel: 0.01

Construction area: 2560 acres (4 sq. mile)

Six paired model and real rainfall sequences from trial number 1 and 2 in Table 6-5 for each assumed highway construction sites (HCS_1, HCS_2, and HCS_3) are used as input data for the VISIOSED model. The result is shown in Table 6-6. A statistical test is performed to see whether these six paired model and real rainfall sequences produce identical soil production in statistical sense.

Table 6-6  Soil Production from Model and Real Sequences

| TSC Type | Site ID | Soil Production from Model Rainfall Sequence (kg) | Soil Production from Real Rainfall Sequence (kg) |
| --- | --- | --- | --- |
| Rock Filter Dam | HCS_1 | 243 | 243 |
|  | HCS_2 | 393 | 372 |
|  | HCS_3 | 55 | 55 |
| Silt Fence | HCS_1 | 398 | 399 |
|  | HCS_2 | 644 | 610 |
|  | HCS_3 | 90 | 90 |

The same unequal t-test is performed here as the one performed in Section 6.3, except that the null hypothesis here shows the mean of soil production from six model rainfall sequences to be the same as that from six real rainfall sequences (H0: (m1-m2) = 0).

The result of this test is shown in Table 6-7. From statistical table (Daniel and Terrell, 1978), for percentiles of the t distribution, ta/2 (t0.05) = 1.645. The null hypothesis is accepted since t = 0.549 (< ta/2 = 1.645) does not lie in the rejection region. Thus, with a = 0.1, there is sufficient evidence to conclude that the means of soil production from the model and the real sequences are equal.

Table 6-7  Statistical Test for Soil Production

| Mean from Model Data | Mean from Real Data |  | Variance from Model Data |  | Variance from Real Data |  | t value |  | df |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 365 |  | 295 |  | 53206 |  | 43659 |  | 0.549 |  | 9 |

6.5 Conclusions from Model Tests

The first model test (Section 6.2) shows that to have a valid mean of precipitation from the SRG model, number of construction days equal to or larger than 60 and number of repeats equal to or larger than 20 are desired. The number of repeats does not show any obvious effect on the result of the SRG model except that it needs to be equal to or larger than 20 to avoid the system error problem in statistical sense.

Based on nine cases from three sites in Texas, rainfall sequences generated from the SRG model and bootstrapping method can not be distinguished from each other by their mean values for a significance level equal of 0.1. Based on six cases from three sites in Texas, the soil productions predicted by rainfall sequences generated from the SRG model and bootstrapping method are also identical to each other for a significance level equal of 0.1.

Chapter 7 Summary and Conclusions

The literature review led to the conclusion that, even though a number of models were developed to simulate rainfall, they are complicated, and difficult to use. There is no specific synthetic rainfall generation model using historical rainfall data for highway construction activities in Texas.

To evaluate the impact of the Storm Water Pollution Prevention Plans on highway construction activities, a simple-to-use rainfall generation model (the SRG model) is constructed to generate a 15-minute rainfall sequence, that can be used to evaluate the soil production and contaminant production associated with highway construction.

The Geographic Information System is used to construct a map of Texas showing all the available rainfall stations. Proposed highway construction sites can be placed on this map, and distances between highway construction sites and selected rainfall stations can be easily found. A Poisson distribution model is found to describe arrival-time distributions very well for raingage stations in Texas. A simulated arrival-time series is generated based upon this Poisson model. A convolution equation is used to generate 15-minute rainfall sequences based on four rainday parameters that preserve total precipitation amount, overall shape, and rain duration of historical data. As the duration and complexity of the historical rainfall patterns on each day increase, the fit between the model and real data deteriorates. This procedure of changing real data to model parameter data saves significant storage space in computer. For daily data, there is a reduction of the data file size by a factor of 30. For 15-minute data, there is a reduction of the data file size by a factor of 10. An inverse distance algorithm is used to combine rainfall sequences from three nearby rainfall stations to generate the rainfall sequence for highway construction sites. A user-friendly interface is built for the SRG model so that user can easily perform data input, calculation and viewing of 15-minute rainfall sequence.

A bootstrapping method was designed to generate 15-minunte rainfall sequence by resampling real 15-minute rainfall data. This option can be executed from the SRG model interface and used to faithfully produce information on intensity from historical  rainfall events.

Parametrically generated rainfall sequences from three assumed highway construction sites are used to compare the effect of the number of construction days and of the number of repeats to run the SRG model on the mean values of precipitation. It is found that number of construction days equal to or larger than 60 and number of repeats equal to or larger than 20 will generate 15-minute rainfall sequences that have good statistical stability of the mean values of precipitation during a construction period.

The mean values of precipitation for rainfall sequences generated from the SRG model and bootstrapping method are indistinguishable from each other when the significance level equals 0.1. The soil productions predicted by rainfall sequences generated from the SRG model and bootstrapping method are identical for a significance level of 0.1.

References

Blaisdell, E.A. (1992) Statistics in Practice, Sanders College Publishing, Fort Worth, CO, 444-448.

Bras, R.L. (1990) Hydrology-An Introduction to Hydrologic Sciences, Addision-Wesley Publishing Company, Reading, MA, 410-415.

Chang, T.J., Kavvas, M.L. and Delleur, J.W. (1984) “Daily Precipitation Modeling by Discrete Autoregressive Moving Average Processes”, Water Resour. Res., Vol. 17, 1261-1271.

Daniel, W.W. and Terrell, J.C. (1978) Business Statistics: Basic Concepts and Methodology, Houghton Mifflin Company, Boston, MA, 629.

Dempster, J. (1993) Computer Analysis of Electrophysiological Signals, Academic Press, San Diego, CA, 169-171.

EarthInfo, Inc. (1995) NCDC Hourly and 15 minute Precipitation (CD), EarthInfo, Inc., Boulder, CO.

Embler, P.F. and Fletcher, M.O. (1981) Effectiveness of Highway Construction on Stream Turbidity and Suspended Solids: A Case Study, Research and Materials Laboratory (Final Report), S.C. Department of Highways and Public Transportation, Columbia, SC.

Foster, G.R., Lane, L.J. (1987) “User Requirements: USDA-Water Erosion Prediction Project (WEPP)”, NSERL Report No. 1, Sept. 1987, USDA-ARS National Soil Erosion Research Lab, Purdue University, W. Lafayette, IN.

Haan, C.T. (1977) Statistical Methods in Hydrology, The Iowa State University Press, Ames, IA.

Hainly, R.A. (1980) “The Effects of Highway Construction on Sediment Discharge into Blockhouse Creek and Steam Valley Run”, Water-resources Investigation 80-68,  Department of Interior, Geological Survey, Harrisburg, PA.

Helm,R.E. (1978) “Sediment Discharge from Highway Construction Near Port Carbon, Pennsylvania”, Water-resources Investigation 80-68, Department of Interior, Geological Survey, Harrisburg, PA.

Jilani, Nooreen (1997) A Rainfall-soil Loss Model for Application to Highway Construction Sites, Master’s Thesis, Department of Civil and Environmental Engineering, University of Houston, Houston, TX.

Lapin, L.L. (1973) Statistics for Modern Business Decisions, Harcourt Brace Jovanovich, Inc, New York, 385-387.

Liu, H. (1997) unpublished work.

Mcclave, J.T. and Benson, G.P. (1994) Statistics for Business and Economics, Dellen, San Francisco, CA.

McCuen, R.H., Snyder, W.M. (1986) Hydrological Modeling - Statistical Methods and Applications, Prentice-Hall, Englewood Cliffs, NJ.

Pattison, A. (1964) “Synthesis of Rainfall Data”, Technical Report No. 40, Department of Civil Engineering, Stanford University, Stanford, CA.

Press, W.H., Flannery, B.P., Teukolsky, S.A., and Vetterling, W.T. (1986) Numerical Recipes-Art of Scientific Computing, Cambridge University Press, Cambridge, UK.

Rodriguez-Iturbe, I., Gupta, V.K., and Waymire, E. (1984) “Scale Considerations in the Modeling of Temporal Rainfall”, Water Resour. Res., Vol. 20, 1911-1619.

Roldan, J. and Woolhiser, D.A. (1982) “Stochastic Daily Precipitation Models, 1, A Comparison of Occurrence Process”, Water Resour. Res., Vol. 18, 1451-1459.

Skees, P.M. and Shenton, L.R. (1974) “Comments on the Statistical Distribution of Rainfall per Period under Various Transformations, Proceedings of the Symposium on Statistical Hydrology, Tucson, AZ, Aug. 31-Sept. 2, 1971”, USDA-ARS, Misc. Pub. No. 1275, June 1974, 172-196.

Slade, J.J., Jr. (1936) “An Asymmetric Probability Function”, Trans. A.S.C.E., Vol. 101, 35-61.

Strategic Mapping Inc. (1994) Atlas GIS Tutorial, Santa Clara, CA.

Tabios III, G.Q. and Salas, J.D. (1985) “A Comparative Analysis of Techniques for Spatial Interpolation of Precipitation”, Water Resour. Bull., Vol. 21 (3), 365-380.

U.S. Federal Register, Vol. 57, No. 175, September 9, 1992, 41176-41356.

Vice, R.B. (1969) “Sediment Movement in an Area of Suburban Highway Construction, Scott Run Basin, Fairfax County, VA, 1941-1964”, UGGS Water -Supply Paper 1591-E.

Yevjevich, V. (1972) Stochastic Process in Hydrology, Water Resources Publications, Fort Collins, CO.