CE 5364 Groundwater Transport Phenomena				March 24, 2005

Continuation of MODFLOW/MODPATH/MOC3D Project

MT3D Tasks

Steady State Hydraulic Conditions

1.  Advection and dispersion.  Use MT3D with L = 10 ft and T = 1 ft and the initial concentrations properly distributed to show the movement of the plume and which pre-existing wells are impacted in terms of earliest and final plume contact.  You may consider the leading and trailing edge of the plume to be represented by 5 or 1 ppb.  Select one and be consistent.

2.  Advection, dispersion, and sorption.  Use a Kd = 0.20 along with the conditions in the previous task.

Demonstrate the plume’s impact on the existing wells similar to the previous task.

3.  Find the necessary half-life with the conditions in problem 2 to prevent the plume from reaching the first pre-existing well.

4.  Use extraction wells to remove the plume. You may use flow rates of 100 gpm or so.

Report Requirements

A.  General Description

Describe the general problem statement with the initial conditions.  Provide a sketch or printout that shows the location and initial concentration values.

B.  Specific Tasks

- Advection only.  Describe the Modpath input necessary to make these happen, then give the answers.  Provide a printout of the result.

- Advection and dispersion.  Describe the input required for MT3D.  Provide a printout of the concentration contours that demonstrate the result.

- Advection, dispersion, and sorption.  Same as previous bullet.

- Advection, dispersion, sorption, and decay.  Same as previous bullet.

- Pumping system.  Describe the approach you used to locate the wells and the Modflow modification.  The following economic data are for your use.

Pumping well

Installation				$50,000

Efficiency				0.80

Annual maintenance			$4000

Energy cost				$0.08/kWh

Outlet elevation at surface		3150 ft

Power to drive one pump

P (kW) = 1.885x10-4 [ Q(gpm) Hlift (ft) ] / e

We can talk more specifically in class about the report requirements as you go.